# Halcyon Meridian 3 - Product Specification

Model year 2026. Internal reference HBW-M3. Status: approved for tooling, November 2025.

# Frame

Double-butted 6061 aluminium, hydroformed top tube, tapered head tube 44 mm to 56 mm. Sizes S, M, L, XL. Weight 1,640 g in size M, painted.

# Drivetrain

1x12, 32T chainring, 10-51T cassette, 12-speed clutch derailleur. Chain line 55 mm.

# Wheels and tyres

29 inch, 30 mm internal rim width, tubeless ready. Front 2.4 inch, rear 2.35 inch.

# Warranty

Frame: lifetime for the first owner. Paint and decals: two years. Bearings: one year.

# Pricing

Target retail price EUR 2,599 (planning figure, November 2025). The list price is set by sales in the January price list and may differ.

| Size | Reach (mm) | Stack (mm) |
| --- | --- | --- |
| S | 425 | 600 |
| M | 445 | 615 |
| L | 465 | 630 |
| XL | 485 | 645 |